Hardware List &

Cost Estimation

PerimeterIQ — The Silent Perimeter: Directional Intrusion System

DSC Blueprints 2026  ·  SVCE  ·  Round 2 — MVP Build

Prepared by: Mohammad Ayman  |  Team PerimeterIQ

QUICK SUMMARY

Total components · total cost · procurement target

| 12 Component Types across 3 categories | ₹620 Estimated Total minimum configuration | ₹780 Recommended Budget with extras + buffer | 1 Day Procurement Time SP Road, Mar 16 |
| --- | --- | --- | --- |

SECTION 01 — SENSING HARDWARE

Core sensors that form the detection layer of the perimeter system

| # | Component | Specification | Qty | Unit ₹ | Total ₹ | Where to Buy |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | LDR Sensor | GL5528, 5–10kΩ dark resistance | 2 | ₹10 | ₹20 | SP Road / Robu.in |
| 02 | Laser Module | 5mW, 650nm red, 3–5V input | 2 | ₹30 | ₹60 | SP Road / Amazon |
| 03 | Resistor | 10kΩ, ¼W (buy 6, use 4) | 6 | ₹1 | ₹6 | Any electronics shop |

NOTE  Buy 6 resistors not 4. Extras cost ₹2 and save a return trip if one burns.

NOTE  Confirm the laser module runs on 3.3V — some need 5V. Check before wiring to ESP32.

SECTION 02 — PROCESSING & CONTROL

Microcontroller, power, and control components

| # | Component | Specification | Qty | Unit ₹ | Total ₹ | Where to Buy |
| --- | --- | --- | --- | --- | --- | --- |
| 04 | ESP32 DevKit | 38-pin, CH340/CP2102 USB-UART | 1 | ₹400 | ₹400 | SP Road / Robu.in |
| 05 | Active Buzzer | 5V active (not passive) | 1 | ₹20 | ₹20 | SP Road / any shop |
| 06 | Tactile Button | 4-pin, 6×6mm momentary | 2 | ₹5 | ₹10 | Any electronics shop |
| 07 | Breadboard | 830 tie-point full size | 1 | ₹80 | ₹80 | SP Road / Amazon |
| 08 | Jumper Wires | Male-to-male, 20cm (pack of 40) | 1 pack | ₹50 | ₹50 | SP Road / Amazon |
| 09 | USB Cable | Micro-USB or USB-C (match ESP32) | 1 | ₹0 | ₹0 | You already have this |

NOTE  Buy ACTIVE buzzer not passive. Active buzzer beeps when you apply voltage. Passive needs a tone signal. Wrong one = no sound.

NOTE  If you already have an ESP32 — skip item 04. If you have a NodeMCU (ESP8266) — it will NOT work. You need ESP32 specifically for dual analog pins and interrupt support.

SECTION 03 — MOUNT & STRUCTURE

Physical housing to maintain exact 10cm sensor spacing — critical for direction accuracy

| # | Component | Specification | Qty | Unit ₹ | Total ₹ | Where to Buy |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Acrylic Strip | 30cm × 3cm × 3mm thick, clear | 1 | ₹80 | ₹80 | Hardware shop / Fab lab |
| 11 | M3 Bolts + Nuts | M3 × 10mm, pack of 10 sets | 1 pack | ₹30 | ₹30 | Hardware shop |
| 12 | Hot Glue / Epoxy | For sensor holder stabilisation | 1 | ₹40 | ₹40 | Any hardware shop |

NOTE  The 10cm spacing must be exact and permanent. Cardboard will not hold alignment after transport. Acrylic + M3 bolts is the minimum viable rigid mount.

NOTE  Drill or mark two points exactly 10cm apart on the acrylic before mounting anything. Measure twice.

COST SUMMARY

Minimum configuration vs recommended budget with buffer

| CATEGORY | MIN ₹ | RECOMMENDED ₹ |
| --- | --- | --- |
| Section 01 — Sensing Hardware | ₹86 | ₹120 |
| Section 02 — Processing & Control | ₹560 | ₹560 |
| Section 03 — Mount & Structure | ₹150 | ₹200 |
| Contingency / Spares | ₹0 | ₹100 |
| TOTAL | ₹620 | ₹980 |

WHERE TO BUY — BANGALORE

Physical stores for same-day purchase on March 16

| STORE | WHAT YOU CAN GET | NOTES |
| --- | --- | --- |
| SP Road, Bangalore | ESP32, LDR, laser, resistors, buzzer, button, breadboard, jumper wires | One-stop. Go here first. Bring cash. |
| VV Puram / Chickpet Hardware | Acrylic strip, M3 bolts, nuts, hot glue | Hardware shops along the lane |
| Robu.in (online) | ESP32, sensors, modules — all in one order | Delivery 1–2 days if ordered today |
| Amazon / Flipkart | Breadboard, jumper wires, USB cables | Backup if SP Road is out of stock |

CRITICAL NOTES BEFORE YOU SHOP

Read all of these before leaving the house

→  ESP32 specifically:  Do NOT buy NodeMCU (ESP8266) by mistake. The ESP8266 has only one analog pin and no dual interrupt support. You need ESP32 Dev Kit specifically — it looks similar. Check the label.

→  Active buzzer not passive:  Active buzzer beeps when you apply 5V directly. Passive buzzer needs a PWM tone signal. They look identical. Ask the shopkeeper specifically for ACTIVE buzzer, or test it in the shop.

→  Laser module voltage:  Some laser modules are 5V only. Some support 3.3V. ESP32 GPIO outputs 3.3V. If your laser module is 5V only, power it from the ESP32 VIN pin (5V from USB), not GPIO. Ask at the shop.

→  LDR GL5528 specifically:  The GL5528 has appropriate sensitivity range for indoor laser detection. Generic LDRs work too but GL5528 is the tested spec. If the shop doesn't have GL5528, any 5mm LDR photoresistor will work.

→  Buy spares for cheap parts:  Resistors: buy 10 not 4. Buttons: buy 3 not 1. They cost ₹5 total extra. If one burns or you miscalculate the threshold, you don't make a second trip.

→  Bring the shopping list printed:  Don't rely on your memory or phone screen in a busy SP Road shop. Print the list on paper. Show it to the shopkeeper. Point at each item.

Budget ₹780. Go to SP Road on March 16. Come home and wire the first LDR the same evening.

Hardware first. Everything else follows.